The difference between the old and new version for Water Conservation Certification of water tank fittings

The new standard:

GB26730-2011《Sanitary ware-Gravity water flushing devices and supports》

The old standard:

CQC3228-2009《Water Conservation Certification Criteria for Fittings for water claset flush tank

1.fill vale

| Test project | The old standard | The new standard |
| --- | --- | --- |
|  | CQC3228-2009《Water Conservation Certification Criteria for Fittings for water claset flush tan》 | GB26730-2011《Sanitary ware-Gravity water flushing devices and supports》 |
| Surface quality | NO | 5.1.1 |
| the coating corrosion resistance for appearance | NO | 5.1.2 |
| Installation and Removal | NO | 5.1.3 |
| Drive mode | NO | 5.1.4 |
| Water Regulation Function | NO | 5.1.5 |
| Thread | NO | 5.2.1 |
| Replenishment Ratio | NO | 5.2.2 |
| Influent Flow | Dynamic pressure＜0.05MPa，Influent Flow≥0.05L/s | Dynamic pressure＜0.05MPa，Influent Flow≥0.05L/s；Dynamic pressure＜0.05MPa，Influent Flow≤0.33L/s |
| Anti-siphon Performance | No siphonage | The permanent CL line identification should be marked in inlet valve. The position for marking of the CL line should not be higher than the measured through siphon proof test |
| Then Open the Function | NO | The inlet valve can automatically open the inlet when the water down to the height for experiment, and automatically close when down to the height for working. 5 consecutive water water level height difference should not exceed 5mm. For continuous injection of water should not be higher than 5mm. |
| Noise | NO | ≤55dB(A) |

Note: The above project need to do the sampling inspection of product (or the report for providing third party inspection by enterprise)

2.flush vale

| Test project | The old standard | The new standard |
| --- | --- | --- |
|  | CQC3228-2009《Water Conservation Certification Criteria for Fittings for water claset flush tan》 | GB26730-2011《Sanitary ware-Gravity water flushing devices and supports》 |
| Surface quality | NO | 5.1.1 |
| the coating corrosion resistance for appearance | NO | 5.1.2 |
| Installation and Removal | NO | 5.1.3 |
| Drive mode | NO | 5.1.4 |
| Water Regulation Function | NO | 5.1.5 |
| Joint Strength | NO | When the joint for threaded connection is subjected to a torque of 14Nm, the thread should be no cracks, no damage. When the other joint is subjected to an axial tension force of 267N, the thread should be no cracks, no damage. |
| Self-closing Seal | The water level is higher 50mm than the remaining and less 5mm than drain valve overflow should not have leakage phenomenon when it closed. | The water level is higher 50mm than the remaining, less 5mm than drain valve overflow and more than half of the remaining water level at 15mm should not have leakage phenomenon when it closed. |
| Overflow capacity | NO | The highest water level should not be higher than the overflow port 20mm. |
| Corrosion resistance for Seal | NO | The changes for dimensions should not be more than 1mm or 5%, weight changes should not be more than 1g or 5%, and no visible physical changes affecting the sealing after the corrosion resistance test. |
| Durability | Making the 50000 times circulation on the dual drainage valve, big gear and small gear. | Making the 100000 times circulation on the dual drainage valve, setting performs an action according to the proportion for 1 times of whole washed and 3 times of half die. |

Note: The above project need to do the sampling inspection of product (or the report for providing third party inspection by enterprise)